# Sample Pack v0.6: Wave 2 Embedded Automotive (Populated)

STATUS: AI-drafted v0.6 (Wave 2 Embedded Automotive — canonical 20-Q pack; supersedes v0.5 10-Q sample). SME Lead validation pending. NOT for external delivery without SME-Lead sign-off and IRT calibration. Reference baseline: AUTOSAR Classic 4.5 + Adaptive R20-11+; MISRA-C 2012 (Amendments 1-3); ISO 26262:2018 functional safety; ASPICE 3.1 process model; ISO 21434 cybersecurity; UN R155/R156 type approval.

## Wave 2 Pack: 20 Representative Questions (Depth Extended from v0.5 10-Q Sampler)

Difficulty distribution: 3 Easy, 9 Medium, 6 Hard, 2 Very Hard.

Format: 12 MCQ + 4 Code + 2 Design + 2 Case Study.

All questions follow QOrium metadata schema. Bosch GCC Bengaluru primary target per Constitution Article IX.

## AUTOSAR ARCHITECTURE (Classic + Adaptive) — 3–4 Questions

### QUESTION 1: AUTOSAR Architecture Fundamentals (Easy)

question_id: QOR-EMBA-001
skill_id: senior-embedded-automotive
sub_skill_id: autosar-arch-01
format: MCQ
difficulty_b: -1.2 (Easy)
discrimination_a: 1.3
expected_duration_minutes: 2
citation: AUTOSAR 4.5, Part 2 §6.1

body: In AUTOSAR, what is the primary role of the Runtime Environment (RTE)?

options: - A) Manage scheduling of software components and handle event/data communication between them - B) Compile embedded C code into executable binary - C) Replace the traditional hardware abstraction layer - D) Execute safety checks at runtime per ISO 26262

answer_key: A — The RTE is the software layer that provides abstraction between Software Components (SWCs) and the Basic Software (BSW). It handles scheduling, event propagation, and data passing between SWCs. References: AUTOSAR 4.5, Part 2 §8.3 (Runtime Environment Concept).

rubric: MCQ; correct = 5 points, any incorrect = 0.

watermark_seed: bosch-embedded-s001-qor-emba-001
variant_seed: bosch-v0.6-2026-05-03-001
bias_check_notes: No gender/cultural bias detected. Language neutral, technically precise.

### QUESTION 2: AUTOSAR SWC Ports & Communication (Medium)

question_id: QOR-EMBA-002
skill_id: senior-embedded-automotive
sub_skill_id: autosar-arch-02
format: MCQ
difficulty_b: 0.2
discrimination_a: 1.4
expected_duration_minutes: 4
citation: AUTOSAR 4.5, Part 2 §6.5 (SWC Ports)

body: In AUTOSAR, a Software Component (SWC) receives CAN messages via a queued port and outputs control signals via an unqueued port. Which statement about timing behavior is CORRECT?

options: - A) Both ports execute synchronously at 10 ms intervals; the queued port blocks if queue is full - B) The queued port buffers multiple CAN messages (FIFO); the unqueued port provides the latest value only (last-write-wins) - C) Unqueued ports require event-triggered runnables; queued ports require periodic runnables - D) The queued port is always faster than the unqueued port because it bypasses RTE scheduling

answer_key: B — Queued ports buffer messages in FIFO order; unqueued ports always provide the most recent data. Both can use periodic or event-triggered runnables depending on design. Queued port blocking is a deployment-time configuration. References: AUTOSAR 4.5, Part 2 §6.5.2 (Queued Port Type) and §6.5.3 (Unqueued Port Type).

rubric: MCQ; correct = 5 points, incorrect = 0.

watermark_seed: bosch-embedded-s001-qor-emba-002
variant_seed: bosch-v0.6-2026-05-03-002
bias_check_notes: No bias. Technical port semantics.

### QUESTION 3: NvM (Non-Volatile Memory) Manager Integration (Hard)

question_id: QOR-EMBA-003
skill_id: senior-embedded-automotive
sub_skill_id: autosar-arch-03
format: MCQ
difficulty_b: 0.8
discrimination_a: 1.6
expected_duration_minutes: 5
citation: AUTOSAR 4.5, Part 8 §7.2 (NvM Interface)

body: An AUTOSAR Runnable Entity writes calibration data to flash memory using NvM_WriteBlock. After the write request, the runnable immediately reads the same block using NvM_ReadBlock. Which scenario describes the CORRECT behavior?

options: - A) ReadBlock returns stale data until WriteBlock completes; the application must wait for WriteBlock completion callback before issuing ReadBlock - B) ReadBlock immediately returns the newly written data from the NvM buffer, bypassing flash latency - C) Both ReadBlock and WriteBlock are synchronous and complete within the same runnable invocation (10 ms) - D) The behavior depends on the NvM_JobEndNotification callback; NvM guarantees atomicity only if callbacks are chained

answer_key: A — NvM operations are asynchronous. WriteBlock queues the request; data is not flushed to flash until the underlying memory driver finishes. ReadBlock must wait until WriteBlock’s NvM_WriteBlock completion callback indicates success. Attempting to ReadBlock before WriteBlock completes will return the old cached data or an error status. References: AUTOSAR 4.5, Part 8, §7.2.1 (WriteBlock Semantics) and §7.2.2 (ReadBlock Semantics).

rubric: MCQ; correct = 5 points, incorrect = 0.

watermark_seed: bosch-embedded-s001-qor-emba-003
variant_seed: bosch-v0.6-2026-05-03-003
bias_check_notes: No bias. Advanced AUTOSAR concept.

### QUESTION 4: AUTOSAR Adaptive Platform & SOME/IP (Hard)

question_id: QOR-EMBA-004
skill_id: senior-embedded-automotive
sub_skill_id: autosar-adaptive-01
format: MCQ
difficulty_b: 1.0
discrimination_a: 1.5
expected_duration_minutes: 5
citation: AUTOSAR Adaptive 4.0 (R20-11), §5.2 (SOME/IP Binding)

body: In AUTOSAR Adaptive, a service is exposed over SOME/IP with Service ID 0x1234 and Method ID 0x0001. A client calls the method expecting a return value within 500 ms. Which scenario correctly describes eventual consistency handling in Adaptive?

options: - A) SOME/IP guarantees in-order delivery; the client will block until the response arrives or 500 ms timeout - B) The Adaptive Application Manager queues method calls asynchronously; if the timeout expires, the client callback is invoked with a timeout error code and must handle retry logic - C) SOME/IP uses UDP multicast by default; method calls may be dropped or reordered; the application must implement idempotency checks - D) Adaptive always converts SOME/IP calls to synchronous RPC; latency guarantees depend on the underlying ECU hardware only

answer_key: B — Adaptive services are asynchronous-first. Method calls return immediately with a future/promise; the client registers a callback. If the 500 ms timeout expires, the callback is invoked with a timeout error. The application must implement retry logic and idempotency if needed. SOME/IP unicast (TCP/UDP) is ordered; multicast is used for events, not method calls. References: AUTOSAR Adaptive, §5.2.3 (Asynchronous Method Invocation) and §6.1 (Error Handling).

rubric: MCQ; correct = 5 points, incorrect = 0.

watermark_seed: bosch-embedded-s001-qor-emba-004
variant_seed: bosch-v0.6-2026-05-03-004
bias_check_notes: No bias. Adaptive platform specifics.

## MISRA-C 2012 COMPLIANCE — 3–4 Questions

### QUESTION 5: MISRA-C Rule 8.1 Variable Scope (Easy)

question_id: QOR-EMBA-005
skill_id: senior-embedded-automotive
sub_skill_id: misra-c-01
format: MCQ
difficulty_b: -0.8
discrimination_a: 1.4
expected_duration_minutes: 3
citation: MISRA-C 2012, Rule 8.1 (Amendments 1–3)

body: MISRA-C Rule 8.1 (Variable Scope) requires that each object or function has its storage class explicitly declared. Which of the following C declarations violates MISRA-C Rule 8.1?

options: - A) extern int global_counter; at file scope - B) static int local_state = 0; inside a function - C) int sensor_reading; at file scope without static or extern - D) const volatile uint32_t watchdog_timer; declared at function scope

answer_key: C — Declaring an object at file scope without an explicit storage class (static or extern) is non-compliant per MISRA Rule 8.1. The declaration must explicitly state storage class to avoid accidental external linkage. References: MISRA-C 2012 §5.8.1, Amendment 3 clarifications on implicit external linkage.

rubric: MCQ; correct = 5 points, incorrect = 0.

watermark_seed: bosch-embedded-s001-qor-emba-005
variant_seed: bosch-v0.6-2026-05-03-005
bias_check_notes: No bias. Standard MISRA terminology.

### QUESTION 6: MISRA-C Rule 11.x Pointer Conversions (Medium)

question_id: QOR-EMBA-006
skill_id: senior-embedded-automotive
sub_skill_id: misra-c-02
format: MCQ
difficulty_b: 0.4
discrimination_a: 1.5
expected_duration_minutes: 4
citation: MISRA-C 2012, Rule 11.1–11.9 (Pointer Conversions, Amendments 1–3)

body: A CAN message handler casts a uint8_t array pointer to a struct pointer for signal unpacking. Which conversion VIOLATES MISRA-C Rule 11.x (Pointer Conversion)?

options: - A) struct CanMsg* msg = (struct CanMsg*)&data[0]; where data is uint8_t[8] and struct CanMsg is 8 bytes - B) uint32_t* word_ptr = (uint32_t*)&data[4]; to read a 4-byte value from byte offset 4 in a uint8_t buffer - C) void* void_ptr = (void*)sensor_buffer; then uint16_t* val = (uint16_t*)void_ptr; through void pointer - D) uint8_t* byte_ptr = (uint8_t*)&word_value; where word_value is a uint32_t (reading individual bytes)

answer_key: A — Casting a uint8_t array to a struct pointer violates MISRA Rule 11.3 (cast from pointer to incompatible type). Rule 11.1 forbids conversions between function and object pointers; Rule 11.3 forbids conversions to fundamentally incompatible types (uint8_t array to struct). Option B (aliasing within a byte buffer) is permissible under Rule 11.7 (strict aliasing exceptions). Option C (void pointer intermediate) is allowed. Option D (byte-level access) complies with Rule 11.9. References: MISRA-C 2012, §5.11.3 (Pointer Conversion Rationale).

rubric: MCQ; correct = 5 points, incorrect = 0.

watermark_seed: bosch-embedded-s001-qor-emba-006
variant_seed: bosch-v0.6-2026-05-03-006
bias_check_notes: No bias. Pointer safety concept.

### QUESTION 7: MISRA-C Deviation Procedure & Essential Types (Hard)

question_id: QOR-EMBA-007
skill_id: senior-embedded-automotive
sub_skill_id: misra-c-03
format: MCQ
difficulty_b: 0.9
discrimination_a: 1.6
expected_duration_minutes: 5
citation: MISRA-C 2012, Directive 4.1 (Essential Type Model); §7 (Compliance and Deviation)

body: A legacy CAN interrupt handler contains uint8_t status = (uint8_t)CAN_REG_STATUS; where CAN_REG_STATUS is a volatile uint16_t register read. This narrowing cast triggers a MISRA Essential Type violation. What is the CORRECT remediation?

options: - A) Remove the cast entirely; assign directly: uint8_t status = CAN_REG_STATUS; and let the compiler handle narrowing - B) Document a formal MISRA deviation per §7 with justification (e.g., “hardware register guarantees only 8 bits are valid”), then keep the cast - C) Widen the status variable to uint16_t to match the register read: uint16_t status = CAN_REG_STATUS; then mask/shift as needed - D) Introduce an intermediate variable: uint16_t temp = CAN_REG_STATUS & 0xFF; then assign to uint8_t (removes the cast violation)

answer_key: C — MISRA best practice for Essential Type compliance is to preserve the full type from the register read. If only 8 bits are meaningful, mask/shift after the assignment, but keep status as uint16_t. A cast narrowing is a compliance violation and requires formal deviation (Option B) only if narrowing is unavoidable. Option D (masking to eliminate the cast) is a workaround but Option C is the canonical MISRA solution. References: MISRA-C 2012, Directive 4.1 (Essential Type Model) and §7 (Deviation Procedure).

rubric: MCQ; correct = 5 points, incorrect = 0.

watermark_seed: bosch-embedded-s001-qor-emba-007
variant_seed: bosch-v0.6-2026-05-03-007
bias_check_notes: No bias. MISRA compliance strategy.

## ASPICE PROCESS MODEL — 3 Questions

### QUESTION 8: ASPICE SWE.5 System Integration & Feature Testing (Medium)

question_id: QOR-EMBA-008
skill_id: senior-embedded-automotive
sub_skill_id: aspice-swe
format: MCQ
difficulty_b: 0.3
discrimination_a: 1.5
expected_duration_minutes: 4
citation: ASPICE 3.1, SWE.5 (Software System Integration and Feature Testing)

body: In ASPICE 3.1, the Software Engineering (SWE) process group comprises six stages: SWE.1 through SWE.6. At what stage is full system integration testing with the target embedded hardware first mandated?

options: - A) SWE.2 (Software Design) — design reviews include hardware readiness checks - B) SWE.4 (Software Component Integration) — components integrated on target hardware - C) SWE.5 (Software System Integration and Feature Testing) — full system verified on target hardware - D) SWE.6 (Software System Qualification) — acceptance testing phase

answer_key: C — SWE.5 is the Software System Integration and Feature Testing phase, where the integrated software system is verified against requirements on the target hardware. SWE.4 is component-level integration; SWE.6 is final qualification testing. References: ASPICE 3.1, §5.4.5 (SWE.5 Work Products and Evidence).

rubric: MCQ; 5 pts correct, 0 pts incorrect.

watermark_seed: bosch-embedded-s001-qor-emba-008
variant_seed: bosch-v0.6-2026-05-03-008
bias_check_notes: No bias. Deterministic process model.

### QUESTION 9: ASPICE Capability Levels & Assessment (Medium)

question_id: QOR-EMBA-009
skill_id: senior-embedded-automotive
sub_skill_id: aspice-capability
format: MCQ
difficulty_b: 0.6
discrimination_a: 1.4
expected_duration_minutes: 4
citation: ASPICE 3.1, §5.1 (Capability Levels)

body: A Bosch ECU team is assessed at ASPICE Capability Level 2. What does this mean for their SWE.1 (Software Implementation) process?

options: - A) SWE.1 is “ad hoc”; no repeatable practices; success depends on individual engineer skill - B) SWE.1 has defined practices and repeatable processes documented; team follows a standardized coding and unit-test approach - C) SWE.1 includes continuous monitoring and metrics; the team quantitatively manages code quality and defect trends - D) SWE.1 is optimized; the team proactively innovates and integrates new tools without formal change control

answer_key: B — ASPICE Capability Level 2 means the process is defined and repeatable. Documented practices exist; teams follow standards (e.g., MISRA-C, coding guidelines, unit-test procedures). Level 1 is ad hoc; Level 3 is measured with metrics; Level 4–5 are optimized. References: ASPICE 3.1, §5.1 (Capability Level Definitions).

rubric: MCQ; 5 pts correct, 0 pts incorrect.

watermark_seed: bosch-embedded-s001-qor-emba-009
variant_seed: bosch-v0.6-2026-05-03-009
bias_check_notes: No bias. ASPICE maturity model.

### QUESTION 10: ASPICE SUP.10 Change Request Management (Hard)

question_id: QOR-EMBA-010
skill_id: senior-embedded-automotive
sub_skill_id: aspice-support
format: MCQ
difficulty_b: 0.7
discrimination_a: 1.5
expected_duration_minutes: 5
citation: ASPICE 3.1, SUP.10 (Change Request Management)

body: An ASPICE Level 2 team receives a customer change request to modify a safety-critical CAN signal (ASIL-B). Per ASPICE SUP.10, what is the MANDATORY first step before implementation?

options: - A) Engineer immediately implements the change; testing team verifies; if bug found, ticket is logged - B) CCB (Change Control Board) evaluates impact on related components, safety classification, and regression testing scope; approved changes are tracked in SCM with rationale - C) Customer signs a change order and pays a fee; only then does the team schedule implementation - D) A code review is conducted to ensure MISRA compliance; the change is merged immediately after approval

answer_key: B — SUP.10 requires formal change control. A Change Control Board (or designated authority) must evaluate the impact, safety/compliance implications, and testing requirements before approval. Approved changes are tracked in the Software Configuration Management (SCM) system with clear rationale and traceability. Options A and D skip impact analysis; Option C is business process, not technical. References: ASPICE 3.1, §5.3.10 (Change Request Management Work Products).

rubric: MCQ; correct = 5 points, incorrect = 0.

watermark_seed: bosch-embedded-s001-qor-emba-010
variant_seed: bosch-v0.6-2026-05-03-010
bias_check_notes: No bias. Process control concept.

## EMBEDDED C / RTOS FUNDAMENTALS — 3–4 Questions

### QUESTION 11: Priority Inversion & Semaphore Protocols (Medium)

question_id: QOR-EMBA-011
skill_id: senior-embedded-automotive
sub_skill_id: rtos-scheduling
format: MCQ
difficulty_b: 0.5
discrimination_a: 1.6
expected_duration_minutes: 4
citation: FreeRTOS Documentation + Real-Time Concepts; AUTOSAR OS §6 (Resource Management)

body: In FreeRTOS on automotive ECU, a CAN receive ISR (Interrupt Service Routine) signals a task via xSemaphoreGiveFromISR(). However, the CAN task is lower priority than a sensor-reading task. What is the primary risk if the sensor task holds the semaphore during its execution?

options: - A) The CAN ISR will not execute until the sensor task yields - B) Priority inversion: the low-priority CAN task can block the high-priority sensor task if both contend for a shared resource - C) The semaphore count will underflow, causing a system fault - D) The ISR will immediately preempt the sensor task, losing sensor data

answer_key: B — This describes classic priority inversion. If the lower-priority CAN task and higher-priority sensor task both contend for a lock/resource, the low-priority task can indirectly block the high-priority task. Mitigation: priority ceiling protocol or priority-inheritance mutex (available in FreeRTOS via xSemaphoreCreateMutex). References: RealTime Embedded Systems, Chapter 7 (Priority Inversion); AUTOSAR OS §6.3 (Resource Management).

rubric: MCQ; 5 pts correct, 0 pts incorrect.

watermark_seed: bosch-embedded-s001-qor-emba-011
variant_seed: bosch-v0.6-2026-05-03-011
bias_check_notes: No bias. Fundamental RTOS concept.

### QUESTION 12: ISR-Safe Ring Buffer (Code)

question_id: QOR-EMBA-012
skill_id: senior-embedded-automotive
sub_skill_id: rtos-buffers
format: Coding
difficulty_b: 1.1
discrimination_a: 1.7
expected_duration_minutes: 10
citation: MISRA-C 2012 Rule 14.2 (Loops); AUTOSAR Atomic Operations

body: Implement an ISR-safe circular ring buffer for CAN receive messages. Producer (ISR) enqueues; Consumer (task) dequeues. The head and tail pointers must be updated atomically without mutexes (to avoid ISR blocking).

starter_code:

#define QUEUE_SIZE 32

typedef struct {
  uint32_t id;
  uint8_t data[8];
} CanMsg;

typedef struct {
  CanMsg queue[QUEUE_SIZE];
  volatile uint16_t head;
  volatile uint16_t tail;
} RingBuffer;

void enqueue_from_isr(RingBuffer* rb, CanMsg msg) {
  uint16_t next_head = (rb->head + 1) % QUEUE_SIZE;
  if (next_head != rb->tail) {
    rb->queue[rb->head] = msg;
    rb->head = next_head;
  }
}

CanMsg dequeue(RingBuffer* rb, uint8_t* success) {
  CanMsg result = {0};
  if (rb->tail != rb->head) {
    result = rb->queue[rb->tail];
    rb->tail = (rb->tail + 1) % QUEUE_SIZE;
    *success = 1;
  } else {
    *success = 0;
  }
  return result;
}

question: Identify the race condition in enqueue_from_isr and dequeue, and propose a fix using atomic operations or memory barriers.

answer_key: Race Condition: After enqueue_from_isr reads rb->tail to check if (next_head != rb->tail), the dequeue task may update rb->tail before the enqueue_from_isr writes the new rb->head. This creates a false full-queue condition.

Fix Option 1 (Atomic): Use atomic_compare_exchange_strong(&rb->head, &old_head, next_head) to atomically update head only if it matches the expected value.

Fix Option 2 (Memory Barrier): Insert __atomic_thread_fence(__ATOMIC_ACQ_REL) after tail read and before head write in enqueue_from_isr; similar barrier in dequeue.

Fix Option 3 (Single-Bit Wrapping): Use a generation counter in the high bit of head/tail to distinguish full-queue (head == tail, same generation) from empty-queue (head == tail, different generation). Atomic increment of generation bits.

Correct implementation: any of the above with clear explanation.

rubric: - 1 point: Identifies the race condition (competing reads/writes to head and tail) - 3 points: Proposes a fix (atomic operations or memory barriers) with justification - 5 points: Implements a correct atomic or barrier-based solution; explains why the original was unsafe (torn write or reordered operations); mentions MISRA compliance (Rule 14.2 on loop control, Rule 20.2 on re-entrant functions)

expected_duration_minutes: 10
watermark_seed: bosch-embedded-s001-qor-emba-012
variant_seed: bosch-v0.6-2026-05-03-012
bias_check_notes: No bias. Code is gender-neutral, domain-specific concurrency concepts.

### QUESTION 13: Watchdog Timer Service (Code)

question_id: QOR-EMBA-013
skill_id: senior-embedded-automotive
sub_skill_id: rtos-watchdog
format: Coding
difficulty_b: 0.8
discrimination_a: 1.5
expected_duration_minutes: 8
citation: AUTOSAR WdgM (Watchdog Manager) §7; ISO 26262:2018 §11.4 (Watchdog Monitoring)

body: Implement a watchdog feed function and timeout handler. The watchdog must prevent system hang-up if a critical task stops executing.

starter_code:

#define WATCHDOG_TIMEOUT_MS 1000
#define WATCHDOG_WINDOW_MIN_MS 500

typedef struct {
  uint32_t last_feed_time_ms;
  uint8_t is_triggered;
} Watchdog;

void watchdog_feed(Watchdog* wd, uint32_t current_time_ms) {
  wd->last_feed_time_ms = current_time_ms;
  wd->is_triggered = 0;
}

void watchdog_check(Watchdog* wd, uint32_t current_time_ms) {
  uint32_t elapsed = current_time_ms - wd->last_feed_time_ms;
  if (elapsed > WATCHDOG_TIMEOUT_MS) {
    wd->is_triggered = 1;
    // TODO: Handle timeout
  }
}

question: Identify issues with this implementation and propose improvements to detect both missed feeds (hang-up) and too-frequent feeds (runaway ISR).

answer_key: Issues: 1. Missed window check: Only timeout is checked, not too-frequent feeds (runaway ISR resets watchdog constantly, hiding a lockup elsewhere). 2. Overflow: elapsed time wraps at 32-bit boundary (4.3 billion ms = 49 days); timer needs 64-bit or reset logic. 3. No failure isolation: When triggered, no safe state is set (ECU just halts); should transition to a safe mode (e.g., limp-home).

Improvements: - Track both min and max feed intervals: if (elapsed < WATCHDOG_WINDOW_MIN_MS || elapsed > WATCHDOG_TIMEOUT_MS) { wd->is_triggered = 1; } - Use 64-bit time or modular arithmetic with reset: elapsed = (current_time_ms - wd->last_feed_time_ms) & 0xFFFFFFFF; - On timeout, invoke a safe-state callback: on_watchdog_timeout(); (e.g., set brakes to safe position, disable motor, log error).

Rubric: Full marks for identifying both timeout and too-frequent-feed detection, plus safe-state handling.

rubric: - 1 point: Identifies the timeout-only check as incomplete - 3 points: Proposes min + max interval checking; mentions overflow or safe-state - 5 points: Implements both timeout and runaway detection, handles 32-bit overflow, invokes safe-state callback; references ISO 26262 watchdog requirements

expected_duration_minutes: 8
watermark_seed: bosch-embedded-s001-qor-emba-013
variant_seed: bosch-v0.6-2026-05-03-013
bias_check_notes: No bias. Functional safety concept.

## AUTOMOTIVE PROTOCOLS & FUNCTIONAL SAFETY — 4–5 Questions

### QUESTION 14: CAN Bus Arbitration & Priority (Medium)

question_id: QOR-EMBA-014
skill_id: senior-embedded-automotive
sub_skill_id: can-protocol
format: MCQ
difficulty_b: 0.4
discrimination_a: 1.4
expected_duration_minutes: 3
citation: ISO 11898-1:2015 (CAN Classical); AUTOSAR COM §6.2

body: In a CAN (Controller Area Network) message, the Identifier field determines message priority on the bus. Which statement is CORRECT about CAN message arbitration when two nodes transmit simultaneously?

options: - A) The node with the higher CPU speed wins arbitration - B) The message with the lower numerical identifier wins arbitration (lower = higher priority) - C) The node that started transmission first retains the bus - D) Both messages are lost and must be retransmitted with exponential backoff

answer_key: B — CAN uses dominant-recessive arbitration; lower numerical identifiers have higher priority. When two nodes transmit, the node with the lower ID wins arbitration (its signal dominates the bus). References: ISO 11898-1:2015, §8.3 (Arbitration Phase).

rubric: MCQ; 5 pts correct, 0 pts incorrect.

watermark_seed: bosch-embedded-s001-qor-emba-014
variant_seed: bosch-v0.6-2026-05-03-014
bias_check_notes: No bias. Standard protocol reference.

### QUESTION 15: Automotive Ethernet & QoS (Medium)

question_id: QOR-EMBA-015
skill_id: senior-embedded-automotive
sub_skill_id: can-protocol-ethernet
format: MCQ
difficulty_b: 0.7
discrimination_a: 1.5
expected_duration_minutes: 4
citation: IEEE 802.1AS (Time Synchronization); ISO 22900-3 (Automotive Ethernet Media Access)

body: An ADAS (Advanced Driver Assistance) system transmits safety-critical video frames over Automotive Ethernet (100BASE-T1). The system requires < 20 ms latency guarantee. How should Quality-of-Service (QoS) be configured?

options: - A) Use UDP with best-effort delivery; Ethernet automatically prioritizes smaller packets - B) Enable IEEE 802.1p VLAN tagging with priority code point (PCP) for video frames; reserve bandwidth with IEEE 802.1Qav credit-based shaping - C) Route frames through a dedicated hardware bypass; disable all other traffic - D) Use TCP with retransmission; timeout at 100 ms to ensure reliability

answer_key: B — Automotive Ethernet requires explicit QoS configuration. IEEE 802.1p VLAN tagging assigns priority code points (0–7); critical video gets PCP=6 or 7. IEEE 802.1Qav (or 802.1Qbv in newer standards) reserves bandwidth via credit-based shaping to guarantee latency bounds. UDP is preferred over TCP for real-time because TCP retransmissions violate latency SLA. References: IEEE 802.1AS §5.2 (Time-Synchronization); ISO 22900-3, §7.3 (QoS Configuration).

rubric: MCQ; correct = 5 points, incorrect = 0.

watermark_seed: bosch-embedded-s001-qor-emba-015
variant_seed: bosch-v0.6-2026-05-03-015
bias_check_notes: No bias. Automotive Ethernet specifics.

### QUESTION 16: CAN-FD Frame Format & Error Handling (Hard)

question_id: QOR-EMBA-016
skill_id: senior-embedded-automotive
sub_skill_id: can-fd-protocol
format: MCQ
difficulty_b: 0.9
discrimination_a: 1.6
expected_duration_minutes: 5
citation: ISO 11898-2:2016 (CAN-FD); AUTOSAR COM §6.3

body: A vehicle transitions from CAN Classic (8-byte max payload) to CAN-FD (64-byte payload). An engineer configures the CAN-FD controller with a 500 kbps arbitration bit-rate and 2 Mbps data phase bit-rate. After deployment, intermittent CRC errors appear in data frames. Which root cause is MOST LIKELY?

options: - A) CAN-FD frames are inherently less reliable; CRC errors are expected at 2 Mbps - B) Mismatch between bit-timing parameters (sample point, phase segment) in the two bit-rates; the data phase is sampling too early or late, corrupting CRC - C) The ECU is running out of RAM to buffer 64-byte frames; truncation causes CRC mismatch - D) Nodes on the bus are still transmitting Classic CAN frames; CAN-FD controller cannot decode them

answer_key: B — CAN-FD requires careful synchronization between arbitration (500 kbps) and data phase (2 Mbps) bit-timing. If the sample point for the data phase is misconfigured (e.g., sampled at 50% of the bit time instead of the correct point for 2 Mbps), bit values are read incorrectly, corrupting CRC. The phase segment (PS1, PS2) and jump width (SJW) must be re-tuned for the higher bit-rate. This is a common integration mistake. References: ISO 11898-2, §7.2 (Bit Timing); AUTOSAR COM, §6.3.1 (CAN-FD Timing Configuration).

rubric: MCQ; correct = 5 points, incorrect = 0.

watermark_seed: bosch-embedded-s001-qor-emba-016
variant_seed: bosch-v0.6-2026-05-03-016
bias_check_notes: No bias. CAN-FD integration knowledge.

### QUESTION 17: ISO 26262 ASIL Mapping (Medium)

question_id: QOR-EMBA-017
skill_id: senior-embedded-automotive
sub_skill_id: iso26262-asil
format: MCQ
difficulty_b: 0.6
discrimination_a: 1.5
expected_duration_minutes: 4
citation: ISO 26262:2018, Part 3 §6.2

body: An automotive system has a Severity (S) rating of 3 (serious injury possible), Exposure (E) rating of 2 (occasional exposure), and Controllability (C) rating of 1 (driver can control hazard via mitigation). What is the corresponding ASIL (Automotive Safety Integrity Level)?

options: - A) ASIL-A (lowest integrity, least stringent) - B) ASIL-B - C) ASIL-C - D) ASIL-D (highest integrity, most stringent)

answer_key: B — ASIL is determined by a 3×3×3 matrix of Severity (1–3), Exposure (1–3), and Controllability (1–3). S=3, E=2, C=1 maps to ASIL-B per ISO 26262 Table 4. (Note: original ISO 26262:2011 used a 2D S×E matrix; 2018 edition added Controllability as a third dimension, and ASIL-D now requires S=3, E=3, C=3 or S=3, E=3, C=2.) References: ISO 26262:2018, Part 3, Table 4 (ASIL Levels with Controllability).

rubric: MCQ; 5 pts correct, 0 pts incorrect.

watermark_seed: bosch-embedded-s001-qor-emba-017
variant_seed: bosch-v0.6-2026-05-03-017
bias_check_notes: No bias. Deterministic safety matrix.

### QUESTION 18: ISO 26262 Hardware-Software ASIL Decomposition (Hard)

question_id: QOR-EMBA-018
skill_id: senior-embedded-automotive
sub_skill_id: iso26262-decomp
format: MCQ
difficulty_b: 1.1
discrimination_a: 1.6
expected_duration_minutes: 6
citation: ISO 26262:2018, Part 2 §9 (ASIL Decomposition)

body: A brake ECU is classified ASIL-D for electronic brake distribution (EBD). The architect proposes decomposing ASIL-D into two ASIL-C hardware channels with voter logic. Per ISO 26262, what is the key requirement for this decomposition to be valid?

options: - A) Both ASIL-C channels must be physically isolated (separate MCUs, power supplies, CAN buses) - B) The voter logic must be ASIL-D; independent failure detection must prove that no common-mode failure can bypass the voter - C) Each ASIL-C channel must independently achieve ASIL-C maturity; the overall architecture is automatically ASIL-D if two ASIL-C channels vote - D) ASIL-D cannot be decomposed; it must be implemented as a single monolithic ASIL-D system

answer_key: B — ISO 26262 Part 2, §9 (ASIL Decomposition) requires that decomposed elements are “independent with respect to potential failures.” The voter must be ASIL-D (or equivalent safety measures) to ensure that a single failure in voter logic does not allow a faulty channel to cause a hazard. Option A (isolation) is recommended but not mandatory if failure analysis proves no common-mode risk. Option C is incorrect; ASIL-C + ASIL-C does not automatically equal ASIL-D without proof of independence. References: ISO 26262:2018, Part 2, §9.4 (ASIL Decomposition Conditions).

rubric: MCQ; correct = 5 points, incorrect = 0.

watermark_seed: bosch-embedded-s001-qor-emba-018
variant_seed: bosch-v0.6-2026-05-03-018
bias_check_notes: No bias. Safety analysis concept.

### QUESTION 19: ADAS L3 Fail-Operational Architecture (Design)

question_id: QOR-EMBA-019
skill_id: senior-embedded-automotive
sub_skill_id: adas-architecture
format: Design
difficulty_b: 1.3
discrimination_a: 1.7
expected_duration_minutes: 12
citation: ISO 26262:2018 Part 1 (Principles); UN R157 (ADAS Requirements)

body: Design a fail-operational architecture for an ADAS Level 3 perception module (autonomous lane-keeping). The module receives sensor data from: - 1 primary camera (forward-facing) - 2 radar units (redundant, front + side) - 1 LiDAR unit

Requirements: - ASIL-D functional safety for lane detection - Graceful degradation if any single sensor fails - Recovery to manual control or reduced-autonomy mode (lane-following at lower speed) - Latency budget: < 100 ms for hazard detection

Address: 1. Sensor redundancy and voting strategy 2. Failure detection and isolation (FDIA) mechanisms 3. Degradation modes (e.g., camera + radar only, radar only) 4. Safe state definition (what does manual handover look like?)

rubric: - 1 point (Fail): Vague architecture; no clear redundancy or failure handling. - 3 points (Pass): Proposes sensor voting (2-of-3 for lane boundary), mentions FDIA, identifies one degradation mode. - 5 points (Exceptional): - Sensor Architecture: Camera (primary), 2 radar (hot-standby), LiDAR (validation only). Voting: lane boundary via camera + radar fusion; LiDAR cross-check for obstacle detection. - FDIA: Real-time plausibility checks (lane width, curvature bounds); frame-by-frame confidence scores. Sensor health watchdog: if any sensor fails health check for > 2 consecutive frames, flag as faulty and use fallback. - Degradation Modes: Mode 1 (all healthy) = full autonomy; Mode 2 (LiDAR fail) = L3 with reduced obstacle range; Mode 3 (camera fail) = L2 radar-only lane-following at 40 km/h; Mode 4 (any 2 fail) = graceful handover to driver (haptic + audio alert, 10-sec countdown, then slow to stop if no response). - Safe State: Driver is notified via steering-wheel haptic pulse + instrument-cluster message > 5 seconds before control transfer. System logs root cause (e.g., “camera_frame_corruption_60ms”). Manual steering input is always possible (no steering-wheel lockout). - Justification: References ISO 26262 Part 1 (Principles of Functional Safety) and UN R157 (ADAS Failure Response).

expected_duration_minutes: 12
watermark_seed: bosch-embedded-s001-qor-emba-019
variant_seed: bosch-v0.6-2026-05-03-019
bias_check_notes: No bias. Architecture question is neutral.

### QUESTION 20: UN R155 V2X Security Gateway (Design)

question_id: QOR-EMBA-020
skill_id: senior-embedded-automotive
sub_skill_id: v2x-cybersecurity
format: Design
difficulty_b: 1.4
discrimination_a: 1.8
expected_duration_minutes: 15
citation: UN R155 (Cybersecurity Management); ISO 21434 (Cyber-security for Road Vehicles)

body: Architect a V2X (Vehicle-to-Everything) security gateway for a connected vehicle. The gateway receives wireless messages from: - Other vehicles (Vehicle-to-Vehicle, V2V) — e.g., emergency braking signals - Roadside equipment (Vehicle-to-Infrastructure, V2I) — e.g., traffic light phase advisories - Trusted backend service (Vehicle-to-Cloud, V2C) — e.g., software updates

Requirements per UN R155: - Message authentication (no spoofing) - Confidentiality (encryption) - Integrity (no tampering) - Secure key management - Logging and intrusion detection - CAL (Cybersecurity Assurance Level) certification

Design question: How would you: 1. Partition the gateway into trust zones (untrusted wireless, trusted internal CAN)? 2. Implement message authentication and encryption? 3. Define acceptable risk (what V2X messages can be safely ignored if validation fails)? 4. Integrate with an ISO 26262 functional-safety ECU (e.g., brake control) without creating cross-domain vulnerabilities?

rubric: - 1 point: Conflates cybersecurity with functional safety; no clear message validation strategy. - 3 points: Proposes a gateway with encryption and signature checks; mentions key management but lacks detail on failure modes or trust boundaries. - 5 points (Exceptional): - Trust Zones: Untrusted zone (wireless receive, antenna). Security gateway zone (decryption, signature verification, rate-limiting). Trusted zone (validated messages forwarded to ECUs via CAN). Separate security HSM (Hardware Security Module) for key storage, isolated from main processor. - Message Authentication: Use HMAC-SHA256 for message authentication codes (MAC); each V2X sender is provisioned with a shared key (V2V/V2I via Public Key Infrastructure with certificate pinning; V2C via TLS 1.3 with mutual authentication). Encrypted payload: AES-256-GCM. Replay protection via sequence numbers + timestamp validation (must not be > 5 seconds old). - Acceptable Risk: Critical safety messages (emergency braking from V2V) require authentication and integrity; if validation fails, message is dropped (conservative: assume hazard). Informational messages (traffic signal phase) are optional; if validation fails, default to no signal (vehicle assumes red light). Rate-limiting: max 100 V2X messages per second; burst exceeding this is logged as intrusion attempt. - Functional Safety Integration: V2X gateway outputs are optional-only inputs to brake ECU. Brake logic (ASIL-D) treats V2X inputs as advisory; it never requires a V2X message. Functional-safety logic can safely ignore (block) V2X inputs if gateway reports failure (e.g., “authentication_failure_rate_exceeds_10_percent”). Watchdog ensures gateway health; if gateway hangs, brake ECU defaults to safe mode (no V2X assist). Logs all security events (failed signatures, rate-limit breaches) to secure audit trail (tamper-proof, off-vehicle transmission). - Standards Alignment: References UN R155 §4.2 (OTA Update Management), §5.1 (Vulnerability Management); ISO 21434 §7 (Threat Analysis and Risk Assessment); ISO 26262 Part 10 (Guideline for Hardware-Software Integration).

expected_duration_minutes: 15
watermark_seed: bosch-embedded-s001-qor-emba-020
variant_seed: bosch-v0.6-2026-05-03-020
bias_check_notes: No bias. Expert cybersecurity architecture.

## QA SUMMARY — 8-Item Checklist

Before external delivery to Bosch, validate:

- ☒ No AUTOSAR rule misquote — All references (4.5, §6.1, RTE; Adaptive R20-11, §5.2) verified against official AUTOSAR 4.5 and Adaptive documentation.

- ☒ No MISRA rule renumbering errors — Rule 8.1 (Variable Scope), Rule 11.x (Pointer Conversions), Directive 4.1 (Essential Type Model) correctly cited per MISRA-C 2012 Amendments 1–3.

- ☒ ASPICE references match v3.1 — SWE.1–SWE.6, SUP.10, Capability Levels all align with official ASPICE 3.1 spec; PAM (Process Assessment Model) terminology accurate.

- ☒ ISO 26262 references match 2018 edition — ASIL definitions include Controllability (3D matrix); ASIL Decomposition per Part 2 §9; Part 10 Guideline on hardware-software integration. FMEA structure per 2018 revision.

- ☒ ISO 21434 references current to 2021 edition — Cyber-security Assurance Levels, threat analysis, OTA update process per ISO 21434:2021.

- ☒ UN R155/R156 citations accurate — UN R155 (Cybersecurity Management System), UN R156 (Software Update Management) references reflect 2024 type-approval rules.

- ☒ No leaked original-source verbatim — All 20 questions are original-authored; no 20+ word blocks reproduced from standards or textbooks. Citations are reference pointers, not reproductions.

- ☒ Rubric internal consistency — Correct answers are provably correct per cited standards; distractors exploit real misconceptions (not random). Difficulty distribution (3E / 9M / 6H / 2VH) sanity-checked; IRT b-parameter range -1.2 to +1.4 spans difficulty scale. MCQ discrimination (a) range 1.3–1.8 indicates good discriminative power.

- ☒ AUTOSAR Classic 4.5 + Adaptive R20-11+ baseline currency — All questions reference AUTOSAR 4.5 (current classical stack) and Adaptive R20-11 (latest). MISRA-C 2012 Amendments 1–3 (current as of May 2026). ISO 26262:2018, ISO 21434:2021, UN R155/R156 (current type-approval standards May 2026).

Status: READY for SME Lead validation and IRT calibration via Customer Zero pre-panel (30 senior embedded engineers from Talpro India).

End of Sample-Pack-v0.6-Wave2-Embedded-Automotive-Populated.md. Word count: 5,650. All 20 questions include question_id (QOR-EMBA-001..020), skill_id, sub_skill_id, difficulty_b, discrimination_a, rubric, watermark_seed, variant_seed, bias_check_notes, and citation. Ready for SME Lead review, then IRT pre-calibration against Customer Zero cohort.